II INSTITUTE OF AERONAUTICAL ENGINEERING

(Autonomous)

Dundigal, Hyderabad - 500 043

Electrical and Electronics Engineering

| Name | B.Akash |
| --- | --- |
| Roll No | 25951A0516 |
| Program | B.Tech |
| Semester | I Semester |
| Class | CSE-A |
| Regulation | R-25 |
| Course Title | Basic Electrical and Electronics Engineering |
| Course Code | AEEE01 |
| AAT-II | Assignment |

Q1) A 200V DC shunt motor takes a total current of 100 A and runs at 750 rpm. The resistance of the armature winding and of shunt field winding is 0.1 and 40 ?, respectively. Compute (i) the torque developed by the armature, and (ii) copper losses. If the friction and iron losses amount to 1500 W, also calculate (iii) shaft power, (iv) shaft torque, and (v) efficiency

Answer:

Given:

- Supply voltage, V=200 V

- Total line current, IL=100 A

- Speed, N=750 rpm

- Armature resistance, Ra=0.1 Ω

- Shunt field resistance, Rsh=40 Ω

- Friction + iron losses = 1500 W

Step 1: Shunt field current

Ish=VRsh=20040=5 A

Step 2: Armature current

Ia=IL-Ish=100-5=95 A

Step 3: Back emf

Eb=V-IaRa
Eb=200-(95×0.1)=200-9.5=190.5 V

(i) Torque developed by armature

Mechanical power developed:

Pdev=EbIa=190.5×95=18097.5 W

Angular speed:

ω=2πN60=2π×75060=78.54 rad/s

Torque developed:

Tdev=Pdevω
Tdev=18097.578.54≈230.5 N\cdotpm

Torque developed = 230.5 N·m

(ii) Copper losses

Armature copper loss:

PCu,a=Ia2Ra=952×0.1=902.5 W

Shunt copper loss:

PCu,sh=VIsh=200×5=1000 W

Total copper loss:

PCu,total=902.5+1000=1902.5 W

Copper losses = 1902.5 W

(iii) Shaft power

Pshaft=Pdev-rotational losses

Rotational losses = friction + iron = 1500 W

Pshaft=18097.5-1500=16597.5 W

Shaft power = 16.597 kW

(iv) Shaft torque

Tshaft=Pshaftω
Tshaft=16597.578.54≈211.5 N\cdotpm

Shaft torque = 211.5 N·m

(v) Efficiency

Input power:

Pin=VIL=200×100=20000 W
η=PshaftPin=16597.520000=0.8299
η≈82.99%

Q2) A 120 V DC shunt motor draws a current of 200 A. The armature resistance is 0.02 Ohm, and shunt field resistance of 30 Ohm Find the back EMF. If the lap-wound armature has 90 slots with 4 conductors per slot, at what speed will the motor run, when the flux per pole is 0.04 Wb?

Answer:

Given:

- Supply voltage, V=120 V

- Line current, IL=200 A

- Armature resistance, Ra=0.02 Ω

- Shunt field resistance, Rsh=30 Ω

- Number of slots = 90

- Conductors per slot = 4

- flux/pole = ϕ=0.04 Wb

- Lap wound → Number of parallel paths, A=P(unknown but will cancel out)

Step 1: Shunt field current

Ish=VRsh=12030=4 A

Step 2: Armature current

Ia=IL-Ish=200-4=196 A

Step 3: Back EMF

Eb=V-IaRa
Eb=120-(196×0.02)
Eb=120-3.92=116.08 V
 Back EMF = 116.08 V

Step 4: Total number of armature conductors

Slots = 90
Conductors/slot = 4

Z=90×4=360 conductors

Step 5: EMF equation for a DC motor

Eb=PϕZN60A

Where

- P=number of poles

- A=Pfor lap winding
→ So P cancels out.

Thus equation becomes:

Eb=ϕZN60

Solve for speed N:

N=60EbϕZ

Substitute values:

N=60×116.080.04×360

Compute denominator:

0.04×360=14.4

Compute numerator:

60×116.08=6964.8

Now:

N=6964.814.4=483.67 rpm

Q3) Classify types of DC generators and explain in detail the various losses of DC generator.

Answer:

Classification of DC Generators

DC generators are classified based on how the field winding is excited.

1. Separately Excited DC Generator

- Field winding is supplied by an external DC source.

- Field current is independent of armature current.

- Used in laboratories and electroplating.

2. Self-Excited DC Generator

In this type, the field winding receives current from the generator’s own armature.

Self-excited generators are further classified into:

(a) Series Wound DC Generator

- Field winding is connected in series with the armature.

- Field current = armature current.

- Produces high current at low speed; used in traction.

(b) Shunt Wound DC Generator

- Field winding is connected in parallel (shunt) with the armature.

- Field current is small and almost constant.

- Used in battery charging and lighting.

(c) Compound Wound DC Generator

Has both series and shunt field windings.

Two types:

- Cumulative Compound

  - Shunt and series fluxes aid each other → stronger field.

  - Used for lighting & power supply.

- Differential Compound

  - Series flux opposes shunt flux → weaker field.

  - Rarely used because it is unstable.

II. Losses in a DC Generator

Losses reduce efficiency. Total losses are classified into four main categories:

1. Copper Losses (I²R Losses)

These occur due to current flowing in the windings.

Types:

(a) Armature Copper Loss

PaCu=Ia2Ra

(b) Field Copper Loss

- Shunt field loss:

Psh=VIsh

- Series field loss:

Pse=Ia2Rse

(c) Brush Contact Loss

Due to voltage drop at brushes (usually 1–2 V).

Copper losses increase with load, so they are variable losses.

2. Iron Losses (Core Losses)

Occur in the armature core due to rotation in the magnetic field.

Two types:

(a) Hysteresis Loss

Due to repeated magnetization/demagnetization.

Ph=ηBmax1.6fV

Reduced by using silicon steel.

(b) Eddy Current Loss

Induced currents in the armature core produce heat.

Pe∝B2f2t2

Reduced by laminating the core.

Iron losses are constant losses (almost independent of load).

3. Mechanical Losses

These include:

(a) Friction Losses

- Bearing friction

- Brush friction

(b) Windage Losses

- Air resistance due to rotating armature

Q4) A full wave bridge rectifier having load resistance of 100? is fed with 220V, Assuming the diodes are ideal, Find the following terms, DC output voltage Rectifier efficiency Peak inverse voltage

Answer:

Peak (maximum) value of the input

Vm=Vrms2=2202≈311.13 V

2) DC (average) output voltage Vdc

For an ideal full-wave bridge,

Vdc=2Vmπ

So

Vdc=2×311.13π≈198.0 V

3) Rectifier efficiency η

Using the usual definition η=PdcVrmsIrms(i.e. DC power delivered divided by the product VrmsIrms) and the standard result for an ideal full-wave rectifier, the maximum theoretical efficiency is

η=8π2≈0.8106≈81.06%

(You can also derive this from Vdc=2Vmπand Irms=Vm/(2R).)

4) Peak inverse voltage (PIV) for each diode

For a bridge rectifier each diode must withstand a reverse voltage equal to the peak of the source:

PIV=Vm≈311.13 V

(Compare: in a centre-tapped full-wave arrangement each diode would see 2VmPIV — but in a bridge each diode sees only Vm.)

- Final answers

- DC output voltage Vdc≈ 198.0 V

- Rectifier efficiency η≈ 81.1 %

- Peak inverse voltage (PIV) ≈ 311.1 V

Q5) For a transistor circuit having ?= 0.98, ICBO = ICO =5 ?A, and IB =100 ?A, find IC and IE.

Answer:

- Current gain (common base):

α=0.98

- Leakage current:

ICBO=ICO=5 μA

- Base current:

IB=100 μA

We must find:

- Collector current IC

- Emitter current IE

Step 1: Use transistor current relation

For a transistor:

IC=αIE+ICBO

Also, from current relation:

IE=IB+IC

Substitute into first equation:

IC=α(IB+IC)+ICBO
IC=αIB+αIC+ICBO

Rearrange:

IC-αIC=αIB+ICBO
IC(1-α)=αIB+ICBO

Step 2: Substitute values

1-α=1-0.98=0.02
IC(0.02)=(0.98)(100μA)+5μA
IC(0.02)=98μA+5μA=103μA
IC=103μA0.02
IC=5150 μA=5.15 mA

Step 3: Find emitter current

IE=IB+IC
IE=100μA+5150μA=5250μA
IE=5.25 mA

Final Answer

| Quantity | Value |
| --- | --- |
| Collector current IC | 5.15 mA |
| Emitter current IE | 5.25 mA |

Q6) Recall base width modulation in bipolar junction transistor.

Answer:

Base width modulation, also called the Early effect, refers to the reduction in the effective base width of a BJT when the collector–base junction reverse bias increases.

Explanation

In a BJT:

- The collector–base junction is reverse biased during normal operation.

- When the collector voltage increases, the depletion region at the collector–base junction widens.

- This widened depletion layer extends into the base region, thereby:

  - Reducing the effective base width

  - Allowing more charge carriers to reach the collector

As a result, the collector current increases slightly even when base current remains constant.

Q7) Sketch the small signal equivalent circuit of CE amplifier?

Answer:

Q8) Sketch the small signal model of BJT Amplifier circuit.

Answer:

Q9) Draw a CB Amplifier and its Hybrid equivalent circuit? Solve the expressions for AIS, AV S and AP for Common Collector amplifier using H parameter model.

Answer:

Small-signal h-parameter model (CE model used in CC connection)

For CE h-model:

- Between base and emitter: input resistance hie

- From collector to emitter: dependent current source hfeib

Since in CC configuration the collector is at AC ground, the current source effectively pushes current into the emitter node.

Approximate relations (neglect hreand hoe):

- ic≈hfeib

- ie=ic+ib≈(hfe+1) ib

Let:

- RL= load at emitter

- RS= source resistance

2.3 Input resistance seen at base

The emitter current flows through RL. The equivalent input resistance at base:

Rin(base)≈hie+(hfe+1) RL

Intuition: the load at emitter is “seen” at base multiplied by the factor hfe1.

If the source has resistance RS, then the total seen by the source is:

Rin(total)=RS+Rin(base)

2.4 Voltage gain AVS=VLVS

- Source voltage divider:
vb=VS⋅Rin(base)RS+Rin(base)

- Emitter voltage (output) is approximately the same as base minus small vbe. Using the h-model:

ve≈vo=(hfe+1)RLhie+(hfe+1)RL vb

So:

AVS=VLVS=voVS≈Rin(base)RS+Rin(base)⏟source divider×(hfe+1)RLhie+(hfe+1)RL⏟local emitter follower gain

In practice, for a good emitter follower:

- (hfe+1)RL≫hie

- Rin(base)≫RS

So:

AVS≈1

(That’s why CC is called a voltage follower.)

2.5 Current gain AIS=ILIS

Let:

- IS= current from source

- IL= current in load RL

Approximate relations:

- Load current: IL=VLRL=voRL≈ie

- Emitter current: ie≈(hfe+1)ib

Source current:

- Source current ISflows into base network and effectively supplies base + emitter currents; but from small-signal viewpoint, majority of that is emitter current (for high h_fe). For approximate analysis, we can write:

AIS=ILIS≈(hfe+1) ibIS

Relating to resistances: with some algebra (treating entire base/emitter network as a load on the source), one gets:

AIS≈(hfe+1) Rin(base)RS+Rin(base)1

In high-gain practical case:

- Rin(base)≫RS

So:

AIS≈(hfe+1)

So the emitter follower gives:

- Large current gain ≈ β1

- Voltage gain ≈ 1

2.6 Power gain AP

By definition:

AP=PoutPin=VLILVSIS=AVS⋅AIS

So:

AP≈AVS×AIS≈1×(hfe+1)≈(hfe+1)

(More accurately, plug in the full expressions for AVSand AISabove.)

Summary for Common Collector (Emitter Follower) using h-parameters

Using CE h-parameters (hie,hfe):

- Input resistance at base:

Rin(base)≈hie+(hfe+1)RL

- Voltage gain (with source):

AVS=VLVS≈Rin(base)RS+Rin(base)⋅(hfe+1)RLhie+(hfe+1)RL≈1(for practical design)

- Current gain:

AIS=ILIS≈(hfe+1)(practically)

- Power gain:

AP=PoutPin=AVS⋅AIS≈(hfe+1)

These are exactly the results examiners like:
CC amplifier → high input resistance, low output resistance, high current & power gain, voltage gain ≈ 1.

Q10) Draw a CE Amplifier and its Hybrid equivalent circuit? Solve the expressions for AI , AV , Zi and AP for Common Emitter amplifier using H parameter model.

Answer:

Deriving AI, AV, Zi, and APfor CE Amplifier

We’ll assume:

- Collector sees an effective AC load RCE=RC∥RL.

- Emitter is AC-grounded (bypassed emitter resistor).

- Use CE h-parameter set: hie,hfe,hre,hoe.

- Input is applied at base; output at collector.

Let:

- i1=input current (base current)

- i2=output current (collector current)

- v1=input voltage (between base and emitter)

- v2=output voltage (collector to emitter; emitter at AC ground)

h-parameter relations (CE):

v1=hiei1+hrev2i2=hfei1+hoev2

At the collector:

v2=vo=-i2RCE

(the minus sign because current entering the top of RCEgives a negative voltage at the collector node w.r.t ground).

2.1 Current gain AI=i2i1

From:

i2=hfei1+hoev2

But v2=-i2RCE, so:

i2=hfei1-hoei2RCE

Bring i2terms together:

i2(1+hoeRCE)=hfei1

So:

AI=i2i1=hfe1+hoeRCE

Simplified (usually in exams): if hoeRCE≪1,

AI≈hfe

(i.e. current gain ≈ transistor β).

2.2 Voltage gain AV=v2v1

We already have:

v2=-i2RCE
v1=hiei1+hrev2

Using the expression for i2:

i2=hfe1+hoeRCEi1

So:

v2=-RCE⋅hfe1+hoeRCEi1=-hfeRCE1+hoeRCEi1

Now v1:

v1=hiei1+hrev2=hiei1+hrehfeRCE1+hoeRCEi1
v1=i1hiehrehfeRCE1+hoeRCE

Thus:

AV=v2v1=-hfeRCE1+hoeRCEi1i1hiehrehfeRCE1+hoeRCE

Cancel i1and simplify:

AV=v2v1=- hfeRCEhie(1+hoeRCE)-hrehfeRCE

Common exam simplification: for most practical BJTs,

- hre≈0

- hoeRCE≪1

Then:

AV≈-hfeRCEhie

or:

AV≈-hfe(RC∥RL)hie

That’s the famous CE amplifier with large AV and 180° phase inversion.

2.3 Input impedance Zi=v1i1

From:

v1=hiei1+hrev2

Using v2=AVv1, but it’s easier to use v2in terms of i1we already had:

v2=-hfeRCE1+hoeRCEi1

Then:

v1=hiei1+hrehfeRCE1+hoeRCEi1
v1=i1hiehrehfeRCE1+hoeRCE

So:

Zi=v1i1=hie-hrehfeRCE1+hoeRCE

Simplified (common exam assumption): hre≈0, so:

Zi≈hie

(i.e. input impedance ≈ transistor’s h_ie.)

If there is a source resistance RSor base bias network, then the total input impedance seen by source will be:

Zin,total≈RS+Zi(or with bias resistors in parallel as appropriate)

2.4 Power gain AP

Power gain:

AP=PoutPin=VoIoViIi=VoViIoIi=AV⋅AI

So, using our expressions:

AP=AV⋅AI= hfeRCEhie(1+hoeRCE)-hrehfeRCE⋅hfe1+hoeRCE

Simplified for exams, with hre≈0,hoeRCE≪1:

AV≈-hfeRCEhie,AI≈hfe

So:

AP≈AV⋅AI≈-hfe2RCEhie

(Sign usually ignored for power gain; magnitude is what matters.)

Exam-style Summary (Common Emitter, using h-parameters)

With RCE=RC∥RLand neglecting hre, hoe:

- Current gain

AI≈hfe

- Voltage gain

AV≈-hfe(RC∥RL)hie

- Input impedance

Zi≈hie

- Power gain

AP≈AV⋅AI≈-hfe2(RC∥RL)hie

These are exactly the forms teachers and examiners expect when they say:
“Solve expressions for AI,AV,Zi, and APof CE amplifier using h-parameter model.”